Step 1: Raw water stored in HDPE storage tanks                           Step 2: NaoCl dosing for Chlorination

Step 3: Raw water passes through Multi-grade filter                  Step 5: Soft water is passed through 150 µ cartridge

Step 4: Further raw water passes through Softener                    filter

Step 6: Soft water then passed through Ultra-

filtration.

Step 7: Dosing of Anti-descalant, NaOH & Sodium metabisulphite done at this stage.

Step 8: Raw water is passed through 5µ filter.

Step 9: Water further passes through double RO membrane.

Step 10:  After RO, the raw water passes through Electro Deionization & converted to Purified water.

Step 11: Purified water now distributed to different user points.