# SYSTEM REQUIREMENT SPECIFICATION

# FOR

# COMPUTER SYSTEM OF

# QC (KARL FISCHER)

| System Name | QC (KARL FISCHER) |
| --- | --- |
| System ID |  |
| Location | Instrument Lab |
| Effective Date |  |

# TABLE OF CONTENTS

1.	PREPARATION AND APPROVALS	3

2.	OBJECTIVE	4

3.	SCOPE	4

4.	SYSTEM DESCRIPTION	4

5.	ROLE AND RESPONSIBILITY	5

6.	REFERENCES	6

7.	DOCUMENTATION PROCEDURE	6

8.	QUALIFICATION COMPLETION AND APPROVAL	7

9.	ACCEPTANCE CRITERIA	7

10.	SYSTEM REQUIREMENTS	8

10.1	Hardware Components	8

10.2	Software Components	8

10.3	Capacity Requirement	8

10.4	Power Utility	9

10.5	Environmental Condition	9

10.6	Communication Link Between Server To Computer System	9

10.7	Window Security	9

10.8	Verification of User Level and Rights	10

10.9	System Response Failure	10

10.10	Electronic Data Security	10

10.11	Audit Trail	11

10.12	Report Generation	11

10.13	Data Back Up	11

10.14	User Prevented From Alternating Date and Time	12

10.15	21 CFR part 11 Clauses	12

10.16	Control Loops Test	12

11.	DISCREPANCIES HANDLING DURING COMPUTER SYSTEM QUALIFICATION	12

12.	DISCREPANCY AND CORRECTIVE ACTION FORM	13

13.	ABBREVIATION	14

# PREPARATION AND APPROVALS:

The signature listed below indicates the preparation and approval of this system requirement specification. This approval is joint responsibility of listed functional areas.

| document development | SIGN / DATE |
| --- | --- |
| Name : __________________ Designation : __________________ |  |

document review and APPROVAL (M/S …………..)

Sign / Date : __________________ Name : __________________ Designation : __________________ Engineering

Sign / Date : __________________ Name : __________________ Designation : __________________ IT

Sign / Date : __________________ Name : __________________ Designation : __________________ Quality Control

document APPROVAL (M/S …………….)

Sign / Date : __________________ Name : __________________ Designation : __________________ Quality Assurance

# OBJECTIVE:

The objective of the system requirement specification defines the requirements of hardware software, functions and documentation for the Computer System (QC_KF) installed at M/s. ………... This document will be cross-referenced with the relevant system specification to ensure that the system specifications have been met according to M/s. ………….. requirements.

# SCOPE:

This document is applicable to validation of Hardware and Software of computer system installed at M/s. …………. Quality control department. This system requirement specification shall define the documentation, references and acceptance criteria to establish that the validation of Hardware and Software of Computer system after modification is installed in accordance with the guidelines laid down by the manufacturer of the system.

# SYSTEM DESCRIPTION:

Computer system of QC_KF defines the controlling of analytical instrument connected to the system. It controlled the process of instrument connected to it by the software installed in the CS. The operator interface is carried out by CS screen. The CS is used to feed required parameters and set points in the system during operation. The system is connected to data server for printing and data backup. The system is secured by IT through password only within the system.

# ROLE AND RESPONSIBILITY:

The validation team comprising of representative from each of the following departments should be responsible for overall compliance with this validation plan.

| Department | Responsibilities |
| --- | --- |
| Validation Agency (……………) | - To collect the necessary data for system requirement specification activities. - To prepare the system requirement specification in coordination with engineering, validation and quality assurance team. - Comply with regulatory / Guidelines / Standards / validation plan requirements throughout the validation life cycle. - To submit system requirement specification for approval. |
| Engineering (M/s. ………….) | - To provide the necessary data for system requirement specification activities. - To review system requirement specification. |
| IT (M/s. …………) | - To provide the necessary data for system requirement specification activities. - To review system requirement specification. |
| Quality Control (M/s. …………) | - To provide the necessary data for system requirement specification activities. - To review system requirement specification. |
| Quality Assurance (M/s. …………) | - To approve and authorized the system requirement specification. |

# REFERENCES:

The publication listed below form part of this reference documents. Each publication shall have latest revision in effect on the date of this document is approved for execution.

| GAMP 5 | Good Automated Manufacturing Practices, Version 5, Guideline Document for Automated Systems from International Society of Pharmaceutical Engineering |
| --- | --- |
| 21 Code of Federal Regulations (CFR), Part 210 | Current Good Manufacturing Practice in Manufacturing, Processing, Packing, or Holding off Drugs; General |
| 21 Code of Federal Regulations (CFR), Part 211 | Current Good Manufacturing Practice for finished Pharmaceuticals |
| 21 Code of Federal Regulations (CFR), Part 11 | 21 Code of Federal Regulations (CFR), Part 11 Electronic Records, Electronic Signatures, Final Rule Electronic Submissions; Establishment of Public Docket, Notice |
| ICH Q9 | International Conference of Harmonization (ICH) quality risk assessment Q9 |
| EU GMP | Laying down the principles and guidelines of GMP in respect of medicinal products for human use. |
| WHO | Appendix 5, validation of computerized systems. |

# DOCUMENTATION PROCEDURE:

- Qualification activities will be performed as defined in the approved document.

- All documentation will be completed during the execution of the qualification.

- Recording of information will be made in permanent ink.

- Fill out complete information in the verification table provided.

- Do not keep any space blank. Mark blank space with a single line throughout the appropriate space with mentioning NA (Not Applicable) and put initial and date.

- Correct the mistakes by drawing a single line through the incorrect data, recording the correct information and then initialing and dating the change.

# QUALIFICATION COMPLETION AND APPROVAL:

- Verify that all tests required by qualification are completed and attached.

- Verify that all amendments and discrepancies are documented, approved and attached.

- If all items in the qualification for the Computer System Based system (QC_KF) have been reviewed and found to be acceptable, sign the corresponding block in the qualification completion and approval form.

# ACCEPTANCE CRITERIA:

- Installation completion as per manufacturer’s recommendations & cGMP requirements.

- Installation of major components as per the design specifications.

- The supply of all necessary documentation from manufacturer.

- The system is operating as intended and is under state of control.

- Operational features meet system requirements and system specifications.

# SYSTEM REQUIREMENTS:

## Hardware Components

| S.No. | Name | Make/ Assemble By | Model No./Specification |  | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Monitor | Acer | - |  | 01 |
| 2 | CPU | Acer | - |  | 01 |
| 3 | Keyboard | Acer | NA |  | 01 |
| 4 | Mouse | Acer | NA |  | 01 |
| 5 | UPS | Emerson Network System | 20 kVA |  | 01 |
| 6 | RAM | Acer | 4 GB |  | 01 |
| 7 | Processor | Intel | I3-6098P 3.60 GHz |  | 01 |
| 8 | Printer | Canon | 3300 |  | 01 |
| 9 | Analytical Instrument | Metrohm | 888 Titrando |  | 01 |
| Communication Port |  |  |  |  |  |
| S.No. | Port Type |  |  | Quantity |  |
| 1 | USB |  |  | 3 |  |
| 2 | Ethernet |  |  | 1 |  |

## Software Components:

| S.No. | Name | Version no. | Quantity |
| --- | --- | --- | --- |
| 1 | Tiamo Light | 2.5 | 01 |
| 2 | Acrobat reader | - | 01 |
| 3 | Windows | 7 64 bit SP1 | 01 |

## Capacity Requirement:

| Local Electronic Storage | No more than 50% of the installed hard disk capacity in PCS components should be consumed by installed software. |
| --- | --- |
| Historical and Archive Storage | Historical data storage capacity should allow for online retrieval of at forever of any historical data. |

## Power Utility:

| S.No. | DESCRIPTION | SPECIFIED |
| --- | --- | --- |
| 1. | Computer System Power Supply | 230 VAC |
| 2. | Analytical instrument | (220-240) VAC |

## Environmental Condition:

| S.No. | DESCRIPTION | TEMPERATURE | RELATIVE HUMIDITY |
| --- | --- | --- | --- |
| 1. | Computer System Environmental Condition | NMT 25 °C | NA |
| 2. | Analytical instrument | NMT 25 °C | NA |

## Communication Link Between Server To Computer System:

| S.No. | DESCRIPTION | Ping with | Pinged Ip |
| --- | --- | --- | --- |
| 1. | Computer system | File Server | 192.168.2.3 |
| 2. | Computer system | Printer | 192.168.2.120 |

## Window Security:

| S.No. | DESCRIPTION | SPECIFIED |
| --- | --- | --- |
| 1. | Login to PC with blank User ID & Blank password. | Access Denied &Error message displayed. |
| 2. | Login to PC with Correct User ID & Blank password. | Access Denied & Error message displayed. |
| 3. | Login to PC with Correct User ID & incorrect password. | Access Denied & Error message displayed. |
| 4. | Login to PC with Incorrect user ID and correct password. | Access Denied & Error message displayed. |
| 5. | Login to PC with correct password. | Access granted |

## Verification of User Level and Rights:

| S.No. | DESCRIPTION | SPECIFIED |
| --- | --- | --- |
| 1 | User Level & Rights | System should be identified login and indiviual rights should be assigned for user. |

## System Response Failure:

| S.No. | DESCRIPTION | SPECIFIED |
| --- | --- | --- |
| 1. | CPU Failure | CPU should be off and monitor cannot be response. |
| 2. | Monitor Failure | Monitor should be off and CPU Should Be On. |
| 3. | UPS Failure | UPS should be off and CPU and Monitor cannot response. |
| 4. | Communication failure between CPU and Monitor | Monitor should not be response. |
| 5. | Communication failure with Local area network | Printing should be stop |
| 6. | Power Failure | UPS supply connected with System to safe shutdown. |

## Electronic Data Security:

| S.No. | DESCRIPTION | SPECIFIED |
| --- | --- | --- |
| 1. | Electronic Record Storage | All the electronic should be store in a correct manner and specified location. |
| 2. | Electronic Data Storage Path Accessbility | Only authorised user shall be access the elecronic storage data. |
| 3. | Access of any other file beside the primary system software | Only qualified and authorized user shall be access other file beside the primary system software. |
| 4. | Electronic Record Maintain | Electronic record shuold maintain in a redundent hard disk / IT server / DVD with specified location. |
| 5. | Print the entire content of electronic records | User should be print the entire content of electroic records. |

## Audit Trail:

| S.No. | DESCRIPTION | SPECIFIED |
| --- | --- | --- |
| 1. | Attempt to login account from authorised user | Login Successful. The same is logged in the audit trail automatically. |
| 2. | Attempt to login account from unauthorised user | Login Fail. The same is logged in the audit trail automatically. |
| 3. | New Account Creation and Deletion | Audit trail should record the creation of new account and deletion. |
| 4. | Password Change | Change in the password shall be logged into the audit trail. |
| 5. | Key Parameter and Setting change during operation from any user | Key Parameter and Setting change during operation from any user should be logged into audit trail. |
| 6. | Editing/Deleting in Audit Trail | No editions/deletion possible in the Audit trail. |
| 7. | Audit Trail Content | Audit trail should have facility to logged the data with time, user identity, reason of change and type of change. |

## Report Generation:

| S.No. | DESCRIPTION | SPECIFIED |
| --- | --- | --- |
| 1. | Report Edition/Deletion | Report shall not be edit/ delete by user |
| 2. | Date and Time stamp on Report during generation/Print | Date and Time stamp on Report during generation/Print. |
| 3. | Redable Formate | Report shall be in human readable format |

## Data Back Up:

| S.No. | DESCRIPTION | SPECIFIED |
| --- | --- | --- |
| 1. | Access to Data Storage Path | System shall have specific Path and limited access for Data Storage. |

## User Prevented From Alternating Date and Time

| S.No. | DESCRIPTION | SPECIFIED |
| --- | --- | --- |
| 1. | User Prevented From Alternating Date and Time | User cannot change or alter the date and time of system. |

## 21 CFR part 11 Clauses:

| S.No. | DESCRIPTION | SPECIFIED |
| --- | --- | --- |
| 1. | 21 CFR part 11 Clauses | System shall be compliance 21 CFR part 11 Clauses |

## Control Loops Test:

| S.No. | DESCRIPTION | SPECIFIED |
| --- | --- | --- |
| 1. | Control Loops Test | System shall be should able to control the set process parameter within the specified limit. |

# DISCREPANCIES HANDLING DURING COMPUTER SYSTEM QUALIFICATION:

- In case of discrepancy observed during qualification, document in the defined column in each table and document the details of the observation in the discrepancy log sheet.

- Inform to engineering, IT, QC and quality assurance about discrepancy.

- Investigate the discrepancy and ensure the possible impact.

- If discrepancy does not have potential to impact on operation as well as performance of the system, close the discrepancy with proper justification.

- The engineering, IT, QC and QA will decide whether discrepancy is acceptable or not.

- If discrepancy is acceptable, provide conclusion and recommendation if any into respective column.

# DISCREPANCY AND CORRECTIVE ACTION FORM:

| Protocol Reference |  |
| --- | --- |
| Discrepancy Number |  |

DISCREPANCY:

| Describe the Discrepancy |  |
| --- | --- |
| Reported by | Date |

CORRECTIVE ACTION:

| Describe corrective action taken (Attach additional sheets if necessary) |  |
| --- | --- |
| Reported by | Date |

DISPOSITION ACTION :

| Acceptable? | Yes | No |  |  |
| --- | --- | --- | --- | --- |
| Discussion |  |  |  |  |
| Approved by |  |  |  | Date |

COMPLETION:

| Completed by | Date |
| --- | --- |

# ABBREVIATION:

| Abbreviations | Description |
| --- | --- |
| GMP | Good Manufacturing Practices |
| CPU | Central Processing Unit |
| RA | Risk Assessment |
| SRS | System Requirement and Specification |
| IQ | Installation Qualification |
| OQ | Operation Qualification |
| PQ | Performance Qualification |
| QA | Quality Assurance |
| TM | Traceability Matrix |
| VSR | Validation Summary Report |
| SOP | Standard Operating Procedure |
| NA | Not Applicable |
| IO | Input Output |
| ICH | International Conference of Harmonization |
| UPS | Uninterruptible Power Supply |
| CS | Computer System |
| NMT | Not More Than |
| WHO | World Health Organization |