## ANALYTICAL METHOD

## VALIDATION PROTOCOL OF

## Itraconazole Capsules BP 200 mg

TABLE OF CONTENTS

| Item No. | Item |
| --- | --- |
| 1.0 | Pre approval |
| 2.0 | Objective |
| 3.0 | Scope |
| 4.0 | Responsibilities |
| 5.0 | Product profile |
| 6.0 | Equipment material needed |
| 7.0 | Analytical Method used |
| 8.0 | Analytical Performance parameters |
| 9.0 | Results |
| 10.0 | Conclusion |

1.0	PRE-APPROVAL:

This review page is the first page of the Protocol for Analytical Method Validation for Assay of Itraconazole in Itraconazole Capsules BP 200 mg by HPLC and is a record of document approval. Signatures below indicate that this document has been reviewed and approved by a representative of various departments and ensures all relevant sections meet requirements.

APPROVED BY:

| QC Manager |  |
| --- | --- |
| MANAGER QA |  |

2.0      Objective:

The aim of the validation of the Analytical method, for Assay of Itraconazole in Itraconazole Capsules BP 200 mg by HPLC, as per the ICH Guidelines, is to ensure that a selected analytical procedure will give reproducible and reliable results.

3.0	Scope:

This document covers the method for Assay of Itraconazole in Itraconazole Capsules BP 200 mg by HPLC. Instrument, location and analyst are used to validate the analytical method as per ICH guidelines.

4.0	Responsibilities:

| S.No. | Department | Designation | Responsibility |
| --- | --- | --- | --- |
| 1. | Quality Control | QC Officer | Preparation of Protocol and to carry out QC test Procedures |
| 2. | Quality Control | Manager QC | Implementation and supervision of protocol. |
| 3. | Quality Assurance | Manager QA | Approval of protocol and review data with QC |

- Product profile:

| - Name of active material | Itraconazole |
| --- | --- |
| - Specifications | BP |
| - Label claim | 200 mg/Capsule |

6.0	Equipment / Material needed:

| For testing (Chemicals) | Testing equipment |
| --- | --- |
| Itraconazole BPCRS | HPLC |
| Hydrochloric acid | Weighing balance |
| Acetonitrile | 0.45 micron filter |
| Orthophosphoric acid | Ultrasonic bath |
| Potassium dihydrogen orthophosphate | Filtration Assembly |
| Water | Glass apparatus |

7.0 Analytical method to be used:

Carry out the method for liquid chromatography, Appendix III D, using the following solutions.

Solution A:  0.1 volumes of hydrochloric acid and 50 volumes of mobile phase.

(1) To a weighed quantity of the oral solution containing 0.1 g of Itraconazole, add 25mL of mobile phase and 0.2mL of hydrochloric acid and shake. Dilute with the mobile phase to produce 100mL with mobile phase and filter through a 0.45-µm PVDF filter, discarding the first 6mL. Dilute 1 volume of the filtrate to 10 volumes with the mobile phase.

(2) 0.01% w/v of Itraconazole BPCRS in solution A.

(3) 0.1% w/v of Itraconazole for system suitability EPCRS in solution A.

Chromatographic Conditions

a) Use a stainless steel column (15 cm x 4.6 mm) packed with octadecylsilyl silica gel for chromatography (5 µm) (Phenomenex Prodigy ODS-2 is suitable).

(b) Use isocratic elution and the mobile phase described below.

(c) Use a flow rate of 2.0mL per minute.

(d) Use a column temperature of 40°.

(e) Use a detection wavelength of 254 nm.

(f) Inject 15 µL of each solution.

(g) Allow the chromatography to proceed for 3 times the retention time of Itraconazole.

Mobile Phase

48 volumes of acetonitrile and 52 volumes of 0.01M potassium dihydrogen orthophosphate, adjusted to pH 3.0 with orthophosphoric acid.

System Suitability

The test is not valid unless, in the chromatogram obtained with solution (3), the peak to valley ratio is at least 2.0, where Hp is the height above the baseline of the peak due to Itraconazole and Hv is the height above the baseline of the lowest point of the curve separating this peak from the peak due to impurity F.

Determination Of Content

Determine the weight per mL of the oral solution, Appendix V G, and calculate the content of Itraconazole, C35H38Cl2N8O4, weight in volume, using the declared content of C35H38Cl2N8O4  in Itraconazole BPCRS.

Calculation:

Sample area  x  wt. of Std. in mg  x 5 x 100  x 50 x  % std. purity as is  x  Ave. filled weight  x  100

-------------------------------------------------------------------------------------------------------------------------

Standard area  x  50  x  50 x  wt. of sample in mg  x 5 x  100  x Label claim

= % Assay

Limit: 90 % to 110 % of label claim

8.0	The following analytical performance parameters are to be carried out.

8.1	System suitability

8.2     Specificity

8.3	Precision

8.3.1  Instrument Precision

8.3.2  Method Precision

8.3.3  Intermediate precision (Ruggedness)

8.4     Accuracy

8.5     Linearity and Range

8.6     Solution Stability

8. 7     Robustness

8.1  System Suitability Parameter:

| Details | Area |  |  |
| --- | --- | --- | --- |
|  | Impurity B | Itraconazole BPCRS | Impurity F |
| Blank |  |  |  |
| Placebo |  |  |  |
| Standard 1 |  |  |  |
| Standard 2 |  |  |  |
| Standard 3 |  |  |  |
| Standard 4 |  |  |  |
| Standard 5 |  |  |  |
| Standard 6 |  |  |  |
| Average |  |  |  |
| SD |  |  |  |
| % RSD |  |  |  |
| Theoretical Plate |  |  |  |
| Tailing Factor |  |  |  |

| Parameter | Specification | Observation of Itraconazole |
| --- | --- | --- |
| Flow Chart | 2.0 ml/min |  |
| Temperature | 40°C |  |
| Wave length | 254 nm |  |
| Theoretical plates | NLT 2000 |  |
| Tailing Factor | NMT 2.0 |  |
| RSD % | NMT 2.0 % |  |

- Acceptance Criteria:

%RSD should not be more than 2.0% for area. Theoretical plate should be NLT 2000, Tailing Factor should NMT 2.0.

Conclusion:

8.2 Specificity:

| Sample ID | RT | Remarks |
| --- | --- | --- |
| Blank |  |  |
| Placebo |  |  |
| Standard |  |  |
| Sample |  |  |

- Acceptance Criteria:

Placebo and Blank should not interfere in the main peak of the Itraconazole.

- Conclusion:

8.3 Precision:

8.3.1 Instrument Precision :

Standard preparation

| No of Injection | Standard Area of Itraconazole |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |
| Average |  |
| SD |  |
| % RSD |  |
| Theoretical Plate |  |
| Tailing Factor |  |

- Acceptance Criteria:

%RSD should not be more than 2.0 % for area and RT. Theoretical plate should be NLT 2000, Tailing Factor should NMT 2.0.

- Conclusion:

8.3.2 Method Precision:

| No of Sample | % Assay of Itraconazole |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |
| Average |  |
| SD |  |
| %RSD |  |

- Acceptance Criteria:

%RSD for %Assay should not be more than 2.0 %

Itraconazole content in Capsules should be within limits (90.0% to 110.0%)

- Conclusion:

8.3.3 Intermediate precision: Ruggedness

For Day 1 the results use data of Instrument precision and method precision.

Day 2, Chemist 2:

Standard preparation

| No of Injection | Standard Area of Itraconazole |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |
| Average |  |
| SD |  |
| % RSD |  |
| Theoretical Plate |  |
| Tailing Factor |  |

Sample Preparation

| No of Sample | % Assay of Itraconazole |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |
| Average |  |
| SD |  |
| %RSD |  |

Conclusion:

8.4 Accuracy

Response of Itraconazole

Standard Preparation

| S.No. | Itraconazole BPCRS |
| --- | --- |
| 1. |  |
| 2. |  |
| 3. |  |
| 4. |  |
| 5. |  |
| 6. |  |
| Mean |  |
| SD |  |
| % RSD |  |

Sample solution:

| Level | Added Conc. (mg) | Area | Qt. Recovered (mg) | % Recovery (Assay) | Av.%Recovery |
| --- | --- | --- | --- | --- | --- |
| 80 % |  |  |  |  |  |
| 80 % |  |  |  |  |  |
| 80 % |  |  |  |  |  |
| 100 % |  |  |  |  |  |
| 100 % |  |  |  |  |  |
| 100 % |  |  |  |  |  |
| 120 % |  |  |  |  |  |
| 120 % |  |  |  |  |  |
| 120 % |  |  |  |  |  |
| Mean |  |  |  |  |  |
| SD |  |  |  |  |  |
| % RSD |  |  |  |  |  |

Conclusion:

8.5   Linearity & Range:

| No. of Injection | Concentration (%) | Concentration (ppm) | Average Area | Area1 | Area2 |
| --- | --- | --- | --- | --- | --- |
| 1 | 70 |  |  |  |  |
| 2 | 80 |  |  |  |  |
| 3 | 90 |  |  |  |  |
| 4 | 100 |  |  |  |  |
| 5 | 110 |  |  |  |  |
| 6 | 120 |  |  |  |  |
| Coefficient Correlation |  |  |  |  |  |

Acceptance Criteria:

Correlation coefficient should not be less than 0.999

- Conclusion:

8.6 Solution Stability:

Standard Preparation

| Time (Hrs) | Injection 1 | Injection 2 | Average Area |
| --- | --- | --- | --- |
| 0 |  |  |  |
| 12 |  |  |  |
| 24 |  |  |  |

Sample preparation

| Time (Hrs) | Area1 | Area2 | % Assay | % Difference |
| --- | --- | --- | --- | --- |
| 0 |  |  |  |  |
| 12 |  |  |  |  |
| 24 |  |  |  |  |

Acceptance Criteria:

% Assay difference should not be more than 2.0 %

Conclusion:

8.7 Robustness:

Determination: Take various samples and analyze it using different Flow rate, different Mobile phase composition as shown in table:

| Sr.No. | Normal | Flow 1.8 ml | Flow 2.2 ml | Mobile phase compo-1 | Mobile phase compo-2 |
| --- | --- | --- | --- | --- | --- |
| Std-1 |  |  |  |  |  |
| Std-2 |  |  |  |  |  |
| Std-3 |  |  |  |  |  |
| Std-4 |  |  |  |  |  |
| Std-5 |  |  |  |  |  |
| Std-6 |  |  |  |  |  |
| Average |  |  |  |  |  |
| SD |  |  |  |  |  |
| %RSD |  |  |  |  |  |
| Sample |  |  |  |  |  |
| Average |  |  |  |  |  |
| % Assay |  |  |  |  |  |
| Difference |  |  |  |  |  |

Flow Rate: 2.0 ± 0.2 ml

Compo-1:  51 volumes of acetonitrile and 49 volumes of 0.01M potassium dihydrogen orthophosphate, adjusted to pH 3.0 with orthophosphoric acid.

Compo-2:  45 volumes of acetonitrile and 55 volumes of 0.01M potassium dihydrogen orthophosphate, adjusted to pH 3.0 with orthophosphoric acid.

- Acceptance Criteria:

Percentage assay difference for each sample should be within 2.0 %.

- Conclusion:

9. RESULT:

| Parameter | Limit |
| --- | --- |
| System Suitability | %RSD should not be more than 2.0% for area and RT. Theoretical plate should be NLT 2000 Tailing Factor should be NMT 2.0 |
| Specificity | Placebo and Blank should not interfere in the main peak of the Itraconazole in Itraconazole Capsules BP 200mg. |
| Precision | Instrument: %RSD should not be more than 2.0% for area and RT. Theoretical plate should be NLT 2000 Tailing Factor should be NMT 2.0 |
|  | Method: %RSD for %Assay should not be more than 2.0 %  Itraconazole content in Capsule should be within limits 90.0 to 110.0% |
| Accuracy/ Recovery | %RSD for %Recovery should not be more than 2.0 %. % Recovery should be between 98.0% to 102.0 % for individual and for all level. |
| Linearity and Range | Correlation coefficient should not be less than 0.999 |
| Solution Stability | %Assay difference should not be more than 2.0 % |
| Ruggedness | %RSD should not be more than 2.0 % for individual and for both the analysts for assay.  Itraconazole content in Capsule should be within limits 90.0 to 110.0% Difference of average assay between two analysts should not be more than 2.0 |
| Robustness | Percentage assay difference for each sample should be within 2.0 % |

10.0 CONCLUSION

| Analysed by | Checked by | Approved by |
| --- | --- | --- |